M04 Chapter 7 Hands On Projects

Hands-On Project 7-1

- Answer questions:

3A.

Client Identifier Field contains option 61

Option length: 7

Hardware type: Ethernet (0x01)

Client MAC address: LiteOnCommun_30:c8:db (00:a0:cc:30:c8:db)

3B.

To verify that the Client Identifier value is the same as the client’s hardware address you can compare it to The Client MAC address in the main body of the DHCP message to see if they match.

3C.

Under Option: 12

Length: 9

Host Name: UTBPOPKI

3D.

Yes the client accepts unicast replies during the boot process. The bootp flag is set to 0x0000 (unicast)

3E.

Option Code 53: DHCP Message TYP

Option Code 61: Client Identifier

Option Code 50: Requested IP Address

Option Code 12: Host Name

Option Code 55: Parameter Request List

Option Code 255: End

Hands-On Project 7-2

- Answer questions:

3A.

Yes, the DHCP client has an IP addres of 10.1.0.3

3B.

DHCP Request (3) is the message type

3C,

The purpose of this packet is to inform the server that the client accepts the offered IP configuration. The client already has an IP so it’s a Lease Renewal.

3D,

No, The client never receives a reply packet.

3E,

The Client is performing a DHCP renew but it receives no reply and then a IGMP leave group message in packet 9 possibly indicating a shutdown or a network disconnect.

4A,

Yes, this DHCP client still has an IP address of 10.1.0.3

4B,

Message type is DHCP Request (3)

4C,

The main difference is the destination address and the message type. Packet 3 is a unicast message sent directly to the server and Packet 5 is a broadcast sent to all devices on the network MAC: ff:ff:ff:ff:ff:ff IP: 255.255.255.255

4D,

No, the client did not receive a reply packet.

4E,

The client request has timed out from the original packet and went unanswered so it is sending out a broadcast request to tyr and find any available DHCP server to extend its lease.

5A,

No, The client does not still have an IP address its set to 0.0.0.0.

5B,

The message type is a DHCP Discover (1)

5C,

Yes, the client eventually receives a reply in packet 11 after the ping.

5D

The client is trying to obtain a DHCP lease.

Hands-On Project 7-3

- Answer questions:

3A.

No the DHCPv6 client does not have a IPv6 global unicast address. The source address has an fe80:: prefix so it’s a Link-Local address.

3B,

The message type is a Solicit (1)

3C.

The host’s name is t6.

3D,

Yes, packet 7 is a direct reply to the solicitation sent in packet 4.

3E,

The client is currently soliciting so its looking for a server.

4A.

The packet number for the final approval is packet 16 wher the reply message happens. The address assigned to the client is 2001:db8:1ab:ba5e::100